Assignment 6

Problem 1:

Hydraulic System

===================================================================

POLE-ZERO ANALYSIS

===================================================================

--- G1(s) Forward TF ---

Index   Pole                            Re(pole)    Stability

------  ------------------------------  ----------  ----------

1       0.000000e+00                    0.0000e+00  MARGINAL

2       -2.091503e+01 +3.314463e+02i    -2.0915e+01  STABLE

3       -2.091503e+01 -3.314463e+02i    -2.0915e+01  STABLE

Index   Zero

------  ------------------------------

1       -7.500000e-01 +8.627717e+00i

2       -7.500000e-01 -8.627717e+00i

--- G2(s) Feedback TF ---

Index   Pole                            Re(pole)    Stability

------  ------------------------------  ----------  ----------

1       -2.091503e+01 +3.314463e+02i    -2.0915e+01  STABLE

2       -2.091503e+01 -3.314463e+02i    -2.0915e+01  STABLE

Index   Zero

------  ------------------------------

1       -7.500000e-01 +8.627717e+00i

2       -7.500000e-01 -8.627717e+00i

Problem 2:

Problem Parameters

------------------

Motor Power            P_motor     = 2000 W

Motor Efficiency       eta_motor   = 0.94

Required Heat Removal  q_Required  = 120.00 W  [= (1 - eta) * P_motor]

Base Temperature       T_b         = 80.0 C

Ambient Temperature    T_inf       = 20.0 C

Temperature Difference T_b-T_inf   = 60.0 C

Convection Coeff       h           = 10.00 W/m^2K

Fin Conductivity       k_fin       = 200.0 W/mK

Fin Width (motor axis) w           = 0.1500 m  (5.9055 in)

Motor Diameter         D           = 0.1500 m  (5.9055 in)

Motor Circumference    Pmax        = 0.4712 m  (18.5527 in)

Min Fin Spacing        smin        = 0.0080 m  (0.3150 in)

Thickness Range        t_range     = [0.0020 : 0.0150] m

Length Range           L_range     = [0.0050 : 0.0250] m

Total Feasible Designs             = 98

Maximum q_fin (feasible designs)   = 4.7894 W

========================================================================

TOP 10 DESIGNS - Ranked by Descending q_fin

========================================================================

Rank   t[m]       t[in]      L[m]       L[in]      N_fins   %Perim   eta_f%   Space[m]   q_fin[W]

------------------------------------------------------------------------------

1      0.0100     0.3937     0.0250     0.9843     26       55.17    99.78    0.0081     4.7894

2      0.0095     0.3740     0.0250     0.9843     26       52.42    99.77    0.0086     4.7739

3      0.0090     0.3543     0.0250     0.9843     26       49.66    99.76    0.0091     4.7583

4      0.0085     0.3346     0.0250     0.9843     26       46.90    99.74    0.0096     4.7427

5      0.0080     0.3150     0.0250     0.9843     26       44.14    99.73    0.0101     4.7270

6      0.0075     0.2953     0.0250     0.9843     26       41.38    99.71    0.0106     4.7113

7      0.0070     0.2756     0.0250     0.9843     26       38.62    99.69    0.0111     4.6954

8      0.0065     0.2559     0.0250     0.9843     26       35.86    99.67    0.0116     4.6794

9      0.0060     0.2362     0.0250     0.9843     26       33.10    99.64    0.0121     4.6632

10     0.0055     0.2165     0.0250     0.9843     26       30.35    99.61    0.0126     4.6468

========================================================================

BEST DESIGN SUMMARY - Rank 1 (Highest q_fin)

========================================================================

Fin Thickness    t       = 0.0100 m   (0.3937 in)

Fin Length       L       = 0.0250 m   (0.9843 in)

Number of Fins   N_fins  = 26

Optimality       mL      = 0.0816

Fin Efficiency   eta_f   = 99.78 %

Perimeter Used   %Perim  = 55.17 %

Fin Spacing      Space   = 0.0081 m   (0.3199 in)

Heat per Fin     q_fin   = 4.7894 W

Total Heat       q_total = 124.5234 W

Problem 3

Problem Parameters

------------------

Motor Power            P_motor     = 2000 W

Motor Efficiency       eta_motor   = 0.94

Required Heat Removal  q_Required  = 120.00 W  [= (1 - eta) * P_motor]

Base Temperature       T_b         = 80.0 C

Ambient Temperature    T_inf       = 20.0 C

Temperature Difference T_b-T_inf   = 60.0 C

Fin Conductivity       k_fin       = 200.0 W/mK

Fin Width (motor axis) w           = 0.1500 m  (5.9055 in)

Motor Diameter         D           = 0.1500 m  (5.9055 in)

Motor Circumference    Pmax        = 0.4712 m  (18.5527 in)

Min Fin Spacing        smin        = 0.0080 m  (0.3150 in)

Thickness Range        t_range     = [0.0020 : 0.0150] m

Length Range           L_range     = [0.0050 : 0.0250] m

Convection Coeff       h_forced    = 32.3959 W/m^2K

Total Feasible Designs             = 499

Maximum q_fin (feasible designs)   = 2.9505 W

Forced Convection Coefficient Calculation

------------------------------------------

Film Temperature       T_f       = 50.0 C

Free Stream Velocity   U_inf     = 10.00 m/s

Air Density            rho_fluid = 1.1270 kg/m^3

Dynamic Viscosity      mu_fluid  = 1.9600e-05 Pa*s

Thermal Conductivity   k_fluid   = 0.0280 W/mK

Specific Heat          cp_fluid  = 1007.0 J/kgK

Reynolds Number        Re_w      = 8.625e+04

Prandtl Number         Pr        = 0.7049

Flow Regime                      = laminar (Re_w <= 5e5)

Nusselt Number         Nu_w      = 173.5495

Forced Convection h    h_forced  = 32.3959 W/m^2K

========================================================================

TOP 10 DESIGNS - Ranked by Descending q_fin

========================================================================

Rank   t[m]       t[in]      L[m]       L[in]      N_fins   %Perim   eta_f%   Space[m]   q_fin[W]

------------------------------------------------------------------------------

1      0.0150     0.5906     0.0250     0.9843     8        25.46    99.51    0.0439     15.9571

2      0.0145     0.5709     0.0250     0.9843     8        24.62    99.49    0.0444     15.9063

3      0.0140     0.5512     0.0250     0.9843     8        23.77    99.48    0.0449     15.8553

4      0.0135     0.5315     0.0250     0.9843     8        22.92    99.46    0.0454     15.8042

5      0.0130     0.5118     0.0250     0.9843     8        22.07    99.44    0.0459     15.7528

6      0.0125     0.4921     0.0250     0.9843     8        21.22    99.42    0.0464     15.7013

7      0.0120     0.4724     0.0250     0.9843     8        20.37    99.40    0.0469     15.6495

8      0.0115     0.4528     0.0250     0.9843     8        19.52    99.37    0.0474     15.5974

9      0.0110     0.4331     0.0250     0.9843     8        18.67    99.35    0.0479     15.5450

10     0.0105     0.4134     0.0250     0.9843     8        17.83    99.32    0.0484     15.4922

========================================================================

BEST DESIGN SUMMARY - Rank 1 (Highest q_fin)

========================================================================

Fin Thickness    t       = 0.0150 m   (0.5906 in)

Fin Length       L       = 0.0250 m   (0.9843 in)

Number of Fins   N_fins  = 8

Optimality       mL      = 0.1219

Fin Efficiency   eta_f   = 99.51 %

Perimeter Used   %Perim  = 25.46 %

Fin Spacing      Space   = 0.0439 m   (1.7285 in)

Heat per Fin     q_fin   = 15.9571 W

Total Heat       q_total = 127.6566 W